GUIDANCE ON THE PROVISION OF KITCHEN & BATHROOM FACILITIES IN HOUSES IN MULTIPLE OCCUPATION.

BATHROOM FACILITIES

1 to 4 persons. There is no requirement for wash hand basins to be provided within sleeping rooms At least one bathroom and one WC must be provided. Please note that the WC can be located within the bathroom. A wash hand basin must be provided along with the WC.

5 persons One wash hand basin must be provided within each sleeping room PLUS At least one bathroom AND one separate WC (with a wash hand basin) must be provided. Please note that if there are two bathrooms in the house and both have WCs (and wash hand basins) located within then this should be considered as satisfactory. In such a circumstance there would be no requirement for a separate WC.

## 6 to 10 persons One wash hand basin must be provided within each sleeping room PLUS At least two bathrooms AND two WCs (with wash hand basins) must be provided. Please note that one of the WCs can be contained within one of the bathrooms, the other WC (and wash hand basin) must be in a separate compartment.

11-15 persons. One wash hand basin must be provided within each sleeping room PLUS At least three bathrooms AND three separate WCs (with wash hand basins) must be provided. Please note that two of the WCs can be contained within two of the bathrooms).

## VENTILATION IN BATHROOM - Mechanical extraction capable of 15 litres/second

## KITCHEN FACILITIES

| BEDSIT TYPE OF LETTING WHERE KITCHEN FACILITIES ARE PROVIDED WITHIN THE LETTING ROOM AND ARE NOT SHARED WITH OTHER TENANTS. | SHARED HOUSE WHERE KITCHEN FACILITIES ARE SHARED WITH OTHER TENANTS. |
| --- | --- |

| PROVISION OF COOKERS |  |
| --- | --- |
| - A suitably sited, “Baby Belling” type cooker (provided with 2 hotplates and a grill) is sufficient. | - 1 standard sized cooker (provided with 4 hotplates and an oven) to be provided within the shared kitchen for every 5 persons occupying the property. Note 1. The addition of a microwave oven within the kitchen would be sufficient to allow the cooking facilities to be used by an additional 2 persons. (In the case of smaller HMOs that would mean 7 persons). If more than 7 persons share the cooking facilities then the addition of a microwave is not acceptable - a second cooker must be provided. Note 2. Where the HMO includes households that consist of more than one person then the cooking facilities should be provided in the ratio of one standard sized cooker for every three lettings. |
| PROVISION OF SINKS |  |
| - A single bowl sink and drainer would be sufficient. Where a sink is provided within each letting then there would be no need to provide additional wash hand basins within the room (See bathroom standards). | - 1 single bowl sink and drainer to be provided within the shared kitchen for every 5 persons occupying the property. Note that a dishwasher can be counted as an additional sink. |

PROVISION OF ELECTRICAL SOCKETS

| - 2 double sockets must be provided within the kitchen area. The above sockets would be in addition to any sockets used for the cooker, dishwasher, refrigerator/freezer, or washing machine. - 2 double sockets must be provided elsewhere within the bedsit for non- kitchen appliances (e.g. TVs, table lighting etc). | - 2 double sockets must be provided within the kitchen area for every 5 persons. The above sockets would be in addition to any sockets used in the shared kitchen for the cooker, dishwasher, refrigerator/freezer, washing machine or microwave oven (if provided to supplement otherwise inadequate cooking facilities). - Elsewhere 2 double sockets must be provided within each habitable room of the property & 1 single socket on the landing and 1 single socket in the hallway. |
| --- | --- |
| PROVISION OF WORKTOPS |  |
| - 1000 x 500 mms of worktop to be provided. Please note that the sink/drainer do not qualify as worktop. | - 2000 x 500 mms of worktop to be provided for every 5 persons occupying the property. Note. Should the number of persons be between 5 and 9 then allow 500mm worktop per additional person. Please note that the sink/drainer do not qualify as worktop. |

| ## STORAGE IN THE KITCHEN |  |
| --- | --- |
| - Cupboard space of a minimum capacity of 0.16m2 per person to be provided | - Cupboard space of a minimum capacity of 0.16m2 per person to be provided |

| ## REFRIGERATED STORAGE |  |
| --- | --- |
| - A “tabletop” refrigerator (approx 2 cu ft capacity) provided within the letting is sufficient. | - A standard sized “under worktop” refrigerator (approx 5 cu ft capacity) to be provided in the kitchen for every 5 persons occupying the property. |

| ## MECHANICAL EXTRACTION IN KITCHEN |  |
| --- | --- |
| - If positioned above hob, mechanical extraction capable of extracting at a rate of 30 litres/second. - If positioned elsewhere in the room, mechanical extraction capable of extracting at a rate of 60 litres/second. | - If positioned above hob, mechanical extraction capable of at a rate of 30 litres/second. - If positioned elsewhere in the room, mechanical extraction capable extracting at a rate of 60 litres/second. |

| HEATING (THROUGHOUT PROPERTY) |  |
| --- | --- |
| - Adequate controllable heating must be provided within the property. Must include fixed, wall-mounted heating (can be radiators or other suitable heaters) in the following locations: - Every bedroom, lounge, dining room or other habitable room. - Every bathroom - Every kitchen - Within general circulation areas (e.g. hallways, landings etc. | - Adequate controllable heating must be provided within the property. Must include fixed, wall-mounted heating (can be radiators or other suitable heaters) in the following locations: - Every bedroom, lounge, dining room or other habitable room. - Every bathroom - Every kitchen - Within general circulation areas (e.g. hallways, landings etc. |